For industry consultation: Proposed harmonised requirements (base level) for Zero Emission Buses in Australia – June 2025

Name:

Organisation:

Contact:

|  | Requirement | Standard reference (where applicable) | Proposed base model (minimum) requirements for Zero Emission Buses in Australia. Note: States may introduce / maintain requirements above the proposed base to address local operating requirements. | Feedback |
| --- | --- | --- | --- | --- |
| 1 | Interoperable charging ports |  | High level base requirement - Two plugs on rear side of bus – one on left side and one on right side. - Vehicle charging inlets shall be CCS2 type. |  |
| 2 | Vehicle design life |  | High level base requirement - Minimum vehicle design life of 21 years. Notes - NSW and SA requirements will be above the base. NSW will remain at 25.9 years. |  |
| 3 | Isolation switches |  | High level base requirement - Driver switch for low voltage and high voltage systems, AND a high and low voltage external switch for emergency and maintenance. |  |
| 4 | Braking systems |  | High level base requirement - Braking system: Regenerative braking, overridden by ABS as required. Brake lights must engage in any capacity. Notes - NSW descent requirements above the base in accordance with TS 00090. |  |
| 5 | Battery warranty |  | High level base requirement - The battery is proposed to be warranted to achieve the required range for a minimum period of 8 years. |  |
| 6 | Bus fire standards | ANCAP ADR 109/00 & /01 | High level base requirement - Battery fire risk detection is harmonised by ADR109/00 & ADR109/01. - Fire barriers between passengers and powertrain batteries shall be provided. Additional guidance - Information for emergency services: The OEM shall provide comprehensive safety and emergency response information to ANCAP for inclusion in the ANCAP rescue smartphone application. |  |
| 7 | Heating, ventilation, air conditioning (HVAC) |  | High level base requirement - 22 degrees with performance pull-down test (from 38 degrees). |  |
| 8 | Approach and departure angles |  | High level base requirement - Underframe skids to be fitted on all four corners. - Minimum approach and departure angles of 7 degrees when fully laden. |  |
| 9 | Entry floor height |  | High level base requirement - Standard height: no more than 410mm – ADR58/00. - Kneeling height no more than 310mm. |  |
| 10 | Minimum range |  | High level base requirement - Range performance requirement: minimum 300km through-out life. - Measure of range: E SORT test (see requirement: energy use reporting). - Range warranty is linked with range performance (see requirement: battery warranty). Additional guidance - Required minimum useable range is 300km throughout the life of bus as determined by the UIPT Standardised On-road Test Cycles, SORT 1 test cycle. - Range is defined as the distance a bus can travel from full charge until the low battery warning on the vehicle’s dashboard appears. |  |
| 11 | Interior fit out |  | High level base requirement - Qld layout configuration (not fit-out) provided as base guideline for costing. Electronic wheelchair ramp not mandatory in the base product. |  |
| 12 | Passenger doors | NSW TS 160 | High level base requirement - Passenger doors shall comply with Transport for NSW Technical Specification 160 – Bus Door Safety Systems Dec 2021. |  |
| 13 | Energy use reporting | UIPT E SORT NSW TS 00091 | High Level base requirement - The energy consumption in kWh/km, usable energy in kWh and range in km shall be provided in accordance with UITP Project ‘SORT’ – Standardised On-road Test Cycles and UITP Project E-SORT – Cycles for electric vehicles. |  |
| 14 | Electrical standards | UN R107 | High level base requirement - All electrical equipment and connections shall conform to UN R107. |  |
| 15 | High voltage safety | UN R100 Rev 3 | High level base requirement - Covered by ADR109/00 and ADR109/01. |  |
| 16 | Advanced Driver Assistance Systems (ADAS) |  | High level base requirement - Base product is not proposed to include ADAS. |  |
| 17 | Passenger counting |  | High level base requirement - Base product is not proposed to include a passenger counting system. |  |
| 18 | No child left on board |  | High level base requirement - No base requirement proposed for a city bus – will become relevant for dedicated school buses |  |
| 19 | Vehicle weight/mass |  | High level base requirement - Performance requirement is to maximise the passenger count. No minimum seated or standing passengers provided for base product. Additional guidance - Note that the standing area must have maximum slope of 8% and be provided with a suitable number of hand straps, handrails and grips as per ADR58/00. - The maximum vehicle weight allowed on the road is 18t as set by the National Heavy Vehicle Regulator. |  |